# TDK Core Selector Parser V1-1 Notebook Summary

Generated: December 3, 2025

# Summary

master parser.

It handles:

- Reading your core list (CSV)

- Downloading PDFs

- Extracting geometry (Ae, le, Ve, etc.)

- Extracting gap, AL, μ, Bs

- Cleaning and normalizing all columns

- Exporting a single unified cleaned core database

➡️ This gives you everything you need for design calculations.

After this TDK_Core_selector_Selector_V1-0.ipynb can be driven

# Overview

This notebook is a comprehensive PDF parser and data extraction tool designed to automatically extract magnetic core geometry parameters from TDK datasheets. It processes CSV files containing part numbers and catalog URLs, downloads/caches PDF datasheets, and extracts critical geometric parameters using intelligent pattern matching and OCR-tolerant parsing techniques.

# Required Input Files

TDK_E_and_Toroid_Cores_no_catalog.csv

Main input CSV file containing TDK core catalog data

Expected Columns:

Part No. - TDK part number/ordering code

Catalog / Data Sheet - URL to PDF datasheet

AL Value / (nH / N²) - Inductance factor value

Saturation Magnetic Flux Density Bs / mT - Saturation flux density

Air Gap / mm - Air gap dimension (if applicable)

Initial Permeability μi - Initial permeability value

Material Name - Ferrite material designation (e.g., N87, N92)

Core Shape or Shape - Core geometry type (E, Toroid, Planar, etc.)

# Output Files

TDK_E_and_Toroid_Cores_no_catalog_cleaned.csv

Enhanced CSV with extracted geometry parameters in SI units

Additional Output:

pdf_cache/ directory - Downloaded PDF datasheets for offline processing and caching

# Detailed Functionality

## 1. Data Loading and Preprocessing

Loads input CSV with TDK core catalog data

Normalizes and cleans material names, core shapes, and part numbers

Converts raw values to SI units:

AL value: nH/N² → H/turn² (×10⁻⁹)

Saturation flux density: mT → T (×10⁻³)

Air gap: mm → m (×10⁻³)

## 2. PDF Download and Caching System

Implements robust HTTP session with automatic retry logic (5 retries)

Downloads PDF datasheets from TDK URLs with proper headers and timeouts

Caches PDFs locally in pdf_cache/ directory to avoid redundant downloads

Smart cache lookup: searches by normalized part number, family code, or filename pattern

Validates PDF content-type and file size (minimum 1KB)

Configurable download delay and failure handling modes

## 3. Intelligent Geometry Extraction

The notebook uses multiple specialized parsers with fallback strategies:

A. Generic E-Core Parser

Extracts magnetic path length (le), effective area (Ae), and volume (Ve)

OCR-tolerant patterns handle spacing variations: "m m" or "mm"

Supports wide number formats: 102 000 | 102,000 | 102000 | 683.5

Focuses on "Magnetic characteristics" section to reduce false matches

Physics-based backfill: calculates missing parameters from relationships (Ve = Ae × le)

Handles lowercase L vs uppercase I confusion in "le" notation

B. E(Large) Family Parser

Specialized parser for TDK E-Large core family with similar pattern matching but tuned for that format

C. Toroid/Ring Core Parser

Detects toroid cores from URL keywords ("ring", "toroid")

Extracts Σl/A (sum of path lengths over area), le, Ae, Ve

Multiple extraction strategies:

Part number matching: searches for exact part code in PDF text

Normalized ordering code lookup (e.g., extracts "B64290L" from full part number)

Page-specific search when part number found on specific page

Header quartet tokenwise fallback: scans for 4 consecutive plausible numbers

D. Planar ER Core Parser

Detects planar/ER cores from shape hints: "ER", "PLANAR", "EER", "EIR"

Also checks URL and part number patterns (e.g., B655xx, B656xx)

Uses similar pattern matching as E-cores but adapted for planar geometry

## 4. Robust Pattern Matching Features

OCR-tolerant: handles spaces in units ("m m" vs "mm"), superscripts (², ³)

Wide number format support: handles grouped digits (102 000), commas (102,000), decimals

Unicode character normalization: minus signs (−, –), non-breaking spaces

Optional punctuation tolerance: "le:" or "le =" or "le" (colon/equals optional)

Case-insensitive matching throughout

Regex-based with DOTALL flag to handle multi-line patterns

## 5. Processing Pipeline

Iterates through each row in input CSV with progress bar (tqdm)

For each core:

Checks if PDF is already cached locally

Downloads PDF if not cached (with retry logic)

Applies parser selection logic based on core type:

Toroid parser for "ring"/"toroid" URLs

Generic E-core parser first attempt

E(Large) parser fallback if generic fails

Planar ER parser if shape hints detected

Final toroid parser attempt if all else fails

Extracts geometry parameters and converts to SI units

Handles exceptions gracefully (logs errors, continues processing)

Joins extracted geometry data back to original DataFrame

Exports enhanced CSV with all original columns plus geometry

## 6. Data Quality Features

Plausibility checks for toroid parameters (ranges: Σl/A: 0.05-500, le: 1-2000 mm, etc.)

Duplicate detection and normalization of part numbers

Error logging for failed extractions

Maintains data integrity: rows without successful extraction get None values

Preserves all original CSV columns in output

# Extracted Geometry Parameters

The notebook adds these columns to the output CSV:

| Parameter | Units | Description |
| --- | --- | --- |
| Σl/A (Sigma_l_over_A_mm_inv) | mm⁻¹ | Sum of path lengths over area (toroids only) |
| le (le_m) | m | Effective magnetic path length |
| Ae (Ae_m2) | m² | Effective cross-sectional area |
| Ve (Ve_m3) | m³ | Effective magnetic volume |

Plus cleaned versions of original columns:

A_L_H_per_turn2 - AL value in H/turn² (SI)

B_s_T - Saturation flux density in Tesla (SI)

gap_m - Air gap in meters (SI)

mu_i - Initial permeability (dimensionless)

material_name - Cleaned material designation

core_shape - Normalized core geometry type

# Configurable Settings

The notebook includes user-adjustable parameters at the top:

INPUT_CSV - Input CSV filename

CLEANED_CSV - Output CSV filename

CACHE_DIR - PDF cache directory (default: pdf_cache/)

SLEEP_BETWEEN - Delay between PDF downloads (default: 0.0 seconds)

STOP_ON_DOWNLOAD_FAILURE - Whether to halt on download errors (default: False)

Column name constants - Customizable CSV column mappings for flexibility

# Technical Implementation Details

PyMuPDF (fitz) for PDF text extraction

Regular expressions (re) for pattern matching

Pandas for DataFrame operations and CSV I/O

Requests library with HTTPAdapter for robust HTTP handling

urllib3 Retry strategy: 5 total retries, exponential backoff

tqdm for progress visualization

NumPy for numerical operations and array handling

# Use Cases and Applications

Automated core selector database population from manufacturer catalogs

Batch processing of hundreds/thousands of TDK core datasheets

Building searchable databases with geometric parameters for inductor/transformer design

Comparing cores across families with standardized SI units

Integration into automated design tools requiring core geometry data

Creating complete core libraries for electromagnetic simulation software

Eliminating manual data entry from PDF datasheets

# Summary

This notebook is a production-ready tool for automated extraction of magnetic core geometry from TDK datasheets. It combines intelligent PDF parsing, multi-strategy fallback logic, OCR tolerance, and robust error handling to process large batches of cores with minimal manual intervention. The output is a cleaned, standardized CSV with SI units ready for use in engineering calculations, database systems, or electromagnetic simulation tools.

────────────────────────────────────────────────────────────────────────────────

Notebook: TDK_Core_selector_Parser_V1-1.ipynb