# TDK Core Selector V1-0 Notebook Summary

Generated: December 3, 2025

# Typical Workflow

1. Run TDK_Core_selector_Parser notebook to generate cleaned CSV

2. Open TDK_Core_selector_Selector notebook

3. Set design requirements: L_target, I_amp, N_max

4. Add cleaned CSV to CLEANED_CSVS list

5. Run notebook to generate winners_100A.csv and plots

6. Analyze plots: balance turns, volume, and core loss

7. Review winners_100A.csv for detailed parameters

8. Select finalist cores based on engineering tradeoffs

9. Verify with manufacturer datasheets

10. Proceed to detailed electromagnetic simulation if needed

# Overview

This notebook is an intelligent core selection and design validation tool that filters and ranks magnetic cores based on engineering requirements. It takes cleaned CSV data (typically from the Parser notebook), calculates required turns and magnetic flux densities, validates designs against saturation limits, and generates comprehensive visualizations with material performance metrics. The tool supports multiple calculation methods, handles gapped and ungapped cores, and provides detailed design analysis for inductor/transformer applications.

# Required Input Files

TDK_E_and_Toroid_Cores_no_catalog_cleaned.csv

Cleaned CSV file with core geometry and parameters (typically output from Parser notebook)

Expected Columns:

Part No. - Core part number/identifier

Ae_m2 - Effective cross-sectional area (m²)

le_m - Effective magnetic path length (m)

Ve_m3 - Effective magnetic volume (m³)

gap_m - Air gap length (m) - optional, can be 0 for ungapped cores

A_L_H_per_turn2 - Inductance factor AL (H/turn²)

mu_i - Initial permeability (dimensionless)

B_s_T - Saturation flux density (Tesla)

Material Name or material_name - Material designation (e.g., N87, N92, N97)

Core Shape or Shape - Core geometry type (E, Toroid, Planar, etc.)

Catalog / Data Sheet - URL or reference to datasheet (optional)

Note: The notebook can process multiple CSV files simultaneously by adding them to the CLEANED_CSVS list.

# Output Files

winners_100A.csv

CSV file containing all cores that meet the design requirements, sorted by needed turns count

Columns in Output:

Source - Original CSV filename

Part No. - Core identifier

Material_Code - Normalized material designation

Needed_Turns - Minimum turns needed to meet inductance target

L_H - Achieved inductance (Henries)

L_uH - Achieved inductance (microHenries)

L_mode - Calculation method used (AL, gap, gap_forced)

B_T - Operating flux density (Tesla)

B_s_T - Saturation flux density (Tesla)

B_ratio - Operating/saturation ratio (safety margin indicator)

Ve_m3 / Ve_mm3 - Effective volume in m³ and mm³

PV_kW_PER_M3 - Material core loss density at reference conditions

All geometry parameters (Ae_m2, le_m, gap_m, etc.)

Visualizations:

Interactive matplotlib plots showing top candidates with multi-axis analysis

# Configurable Design Parameters

User-adjustable settings at the top of the notebook:

| Parameter | Default Value | Description |
| --- | --- | --- |
| L_target | 3.5 µH | Target inductance value |
| N_max | 10 | Maximum allowed turns |
| I_amp | 100 A | Operating current for B-field calculation |
| PLOT_LIMIT | 999 | Maximum number of cores to display in plots |
| FORCE_MU_EFF | False | Force use of µ_eff calculation even when AL available |
| USE_AL_DEFAULT | True | Prefer AL value when available (recommended) |
| SHOW_MATERIAL_PV | True | Display material core loss density (Pv) overlay |

# Detailed Functionality

## 1. Data Loading and Validation

Loads one or more cleaned CSV files with core data

Automatically detects column names with flexible naming (Core Shape vs core_shape, etc.)

Converts all parameters to numeric format with error handling

Extracts and normalizes material codes (N87, N92, T38, etc.) using regex patterns

Validates each row for required parameters based on calculation method

Provides detailed skip messages for invalid rows with reasons

## 2. Intelligent Inductance Calculation

The notebook supports multiple calculation methods with automatic fallback:

Method A: AL Factor (Preferred Default)

Uses datasheet inductance factor: L = AL × N²

Most accurate when available (includes manufacturer tolerances)

Automatically handles gapped core AL values

Fast and simple calculation

Method B: Effective Permeability (µ_eff)

Calculates from first principles using core geometry and gap:

µ_eff = 1 / [(1/µᵢ) + (gap/le)]

L = µ₀ × µ_eff × N² × Ae / le

Supports ungapped cores (gap = 0, µ_eff = µᵢ)

Required parameters: Ae, le, µᵢ, gap

Useful when AL not specified or for custom gaps

Calculation Priority Logic:

If FORCE_MU_EFF = True: Always use µ_eff method when geometry available

If USE_AL_DEFAULT = True (default): Prefer AL when available

Fallback: Use µ_eff if AL missing but geometry available

Each result tagged with calculation mode for traceability

## 3. Turn Count Optimization

Iterates N from 1 to N_max for each core

Calculates inductance for each turn count

Finds minimum N where L ≥ L_target

Stops at first valid solution (efficiency)

Marks rows as "Meets_L" when target achieved

## 4. Magnetic Flux Density Validation

Calculates peak flux density and validates against saturation:

B = (L × I) / (N × Ae)

Uses operating current (I_amp) from settings

Compares to datasheet saturation flux density (B_s)

Calculates B_ratio = B / B_s (safety margin indicator)

Marks rows as "Meets_B" when B ≤ B_s

Typical design target: B_ratio ≤ 0.5-0.8 for temperature margin

## 5. Material Core Loss Analysis

Incorporates TDK material power loss data:

Built-in lookup table with Pv (kW/m³) for 20+ TDK materials

Reference conditions: 100 kHz, 200 mT, 100°C

Materials included: N27, N41, N51, N87, N88, N92, N95, N96, N97, and more

Overlays Pv on volume plots to identify high-loss materials

Enables quick assessment of thermal performance

Unknown materials default to 0 (marked for user investigation)

## 6. Winner Filtering and Ranking

Applies comprehensive filtering to identify viable designs:

Must meet inductance target: L ≥ L_target

Must avoid saturation: B ≤ B_s

Must fit within turn count limit: N < N_max

Must have all required parameters available

Sorts by needed turns (lower is better for window area)

Secondary sort by part number for consistency

## 7. Advanced Visualization

Generates publication-quality dual-panel plots:

Top Panel: Turns and Inductance Analysis

Bar chart: Needed turns (N) for each core

Color-coded by core shape (E-core, Toroid, Planar, etc.)

Hatching pattern indicates gapped cores (///) vs ungapped (solid)

Right y-axis: Red line showing achieved inductance (µH)

Dual legends: Core shapes + Gap status

Custom y-tick spacing for readability

Bottom Panel: Volume and Core Loss

Bar chart: Effective volume (Ve) in mm³

Same color coding as top panel (core shape)

Right y-axis: Black line showing material Pv (kW/m³)

Enables size vs. loss tradeoff analysis

Shared x-axis: Part numbers (rotated for space)

Integrated legends with material properties

Plot Customization:

Adjustable y-tick step sizes (YTICK_STEP_N, YTICK_STEP_L, etc.)

Plot limit to avoid overcrowding (PLOT_LIMIT)

Automatic color palette (20 distinct colors from tab20)

Grid lines and transparent overlays for clarity

Tight layout for professional appearance

# Row Validation Requirements

Each core must meet one of these parameter combinations:

For Inductance Calculation (one of):

Option 1: AL value available (A_L_H_per_turn2 > 0)

Option 2: All geometry parameters (Ae > 0, le > 0, µᵢ > 0, gap ≥ 0)

For B-Field Validation (both required):

Effective area: Ae_m2 > 0

Saturation flux density: B_s_T > 0

Invalid Row Handling: Rows missing required parameters are skipped with detailed console messages explaining which parameters are missing. This prevents errors and provides actionable feedback.

# Built-in Material Core Loss Reference

Volumetric core loss (Pv) at 100 kHz, 200 mT, 100°C:

| Material | Pv (kW/m³) |
| --- | --- |
| N27 | 920 |
| N41 | 1400 |
| N51 | 450 |
| N87 | 375 |
| N88 | 430 |
| N92 | 410 |
| N95 | 350 |
| N96 | 390 |
| N97 | 300 |
| N91 | 600 |
| N72 | 540 |

Note: Materials not listed default to 0 and require manual lookup from datasheets. Pv values are manufacturer-specified at standardized test conditions.

# Technical Implementation

Pandas for efficient DataFrame operations and CSV I/O

NumPy for numerical computations and array operations

Matplotlib for publication-quality visualizations

Regex (re) for material code extraction and normalization

Physics constants: µ₀ = 4π × 10⁻⁷ H/m

Vectorized operations for performance on large datasets

Robust NaN handling throughout calculations

Type-safe numeric conversions with error handling

Flexible column detection for varied input formats

# Summary

The TDK Core Selector V1-0 notebook is a comprehensive design analysis and validation tool for magnetic component selection. It bridges the gap between raw datasheet data and practical engineering decisions by calculating required turns, validating flux densities, ranking cores by multiple criteria, and presenting results through intuitive visualizations. The tool supports flexible calculation methods, handles both gapped and ungapped cores, incorporates material loss data, and generates production-ready output files. It is an essential tool for power electronics engineers working with TDK magnetic components.

────────────────────────────────────────────────────────────────────────────────

Notebook: TDK_Core_selector_Selector_V1-0.ipynb