# TDK Core Data to CST Notebook Summary

Generated: December 3, 2025

# Overview

This notebook prepares magnetic core material data (specifically TDK N87 ferrite) for use in CST Studio Suite electromagnetic simulations. It performs three main data processing and modeling tasks to convert raw material characterization data into CST-compatible formats.

# Required Input Files

The notebook requires two Excel files as input:

N87_Permeability.xlsx - Contains complex permeability data (μ′ and μ″) vs frequency

Expected columns: Series1.X, Series1.Y (μ′ data), Series2.X, Series2.Y (μ″ data)
Note: Frequency data in kHz, converted to Hz

N87_BH.xlsx - Contains B-H hysteresis curve data

Expected columns: X (H field in A/m), Y (B field in mT)

# Cell 1: Permeability Data Interpolation

Purpose:

Process complex permeability data for CST compatibility

Input File:

N87_Permeability.xlsx

Processing Steps:

Loads μ′ (real permeability) and μ″ (imaginary permeability) from separate data series

Converts frequency from kHz to Hz (SI units)

Interpolates μ′ onto the same frequency grid as μ″ for consistency

Handles edge cases: extends first value below range, sets to 0 above maximum frequency

Removes invalid data points (NaN, non-positive frequencies)

Output File:

N87_Permeability_interp_FIX2.txt (tab-separated format: frequency [Hz], μ′, μ″)

Visualization:

Generates plots showing original data points and interpolated curves on log-frequency scale

# Cells 2-4: B-H Magnetization Curve Modeling

Purpose:

Generate initial magnetization curves using analytical models for CST saturation effects

Input File:

N87_BH.xlsx

Processing Steps:

Extracts clean first-quadrant, monotonic B-H data from raw measurements

Estimates material parameters: initial permeability (μᵢ) from low-field slope

Estimates saturation polarization (Jₛ) from high-field asymptote

Sweeps through different "c" parameter values (shape parameter: 2.0 to 3.0, 5 values)

Applies analytical model: B(H) = μ₀H + Jₛ·x/(1+x^c)^(1/c), where x = H/H₀

Cell 4 allows manual selection of optimal c value with visualization

Output Files:

Multiple files: initial_BH_c_X.XXX.txt in out/ directory (one per c value)

Parameters Estimated:

μᵢ (initial permeability) - from low-field slope fit

Jₛ (saturation polarization) - from high-field behavior

c (shape parameter) - swept to find best curve match

# Cell 5: Advanced Havriliak-Negami Dispersive Model Fitting

Purpose:

Fit complex permeability data to CST's Generalized Debye (Havriliak-Negami) dispersive model with comprehensive parameter optimization

Input File:

N87_Permeability.xlsx

Optimization Process:

Auto-tunes a single-term HN model through extensive parameter sweeps:

Different frequency bands (fit ranges) - 4 configurations

Multiple μ″ weighting factors (2.0, 3.0, 4.0, 5.0)

Alpha parameter modes: frozen at 0.999 or free optimization

Beta parameter modes: frozen at 1.0 or free optimization

Multiple restart strategies for robustness - 3 configurations

Uses sophisticated least-squares optimization with soft L1 loss function

Evaluates hundreds of parameter combinations to find optimal fit

Model Parameters:

μ∞ (high-frequency permeability limit)

μₓ (static/low-frequency permeability)

τ (relaxation time in seconds)

α (alpha parameter, CST-safe: < 1.0)

β (beta parameter, ≤ 1.0)

Output Files:

N87_Permeability_User_For_CST.txt - CST User format data (frequency, μ′, μ″)

N87_HN_params_best.csv - Best-fit HN parameters (μ∞, μₓ, τ, α, β)

N87_HN_leaderboard.csv - All trial results ranked by optimization score

N87_HN_fit_best.png - Visualization comparing best fit vs measured data

N87_HN_fit_errors.png - Error analysis plot (log scale, absolute % errors)

Error Analysis & Tolerance Estimates:

Inductance tolerance (from μ′ error) - median and maximum % errors

Core loss/ESR tolerance (from μ″ error) - median and maximum % errors

Quality factor (Q) error analysis

Suggested loss scale factors for temperature rise/efficiency bracketing

Frequency-dependent error plots with reference lines at 1%, 5%, 10%

# Summary of Outputs

The notebook generates all necessary files for CST material definition:

| Output Type | Files Generated |
| --- | --- |
| Permeability Data | N87_Permeability_interp_FIX2.txt, N87_Permeability_User_For_CST.txt |
| B-H Curves | Multiple initial_BH_c_X.XXX.txt files |
| Dispersive Model | N87_HN_params_best.csv, N87_HN_leaderboard.csv, visualization plots |

# Usage in CST Studio Suite

Import permeability data via CST → Materials → User-defined dispersive material

Paste HN parameters (μ∞, μₓ, τ, α, β) into Generalized Debye settings

Enable Field dependency → B-H curve and import appropriate initial_BH file for saturation modeling

Use loss scale factors to adjust for temperature effects if needed

The optimized parameters ensure accurate electromagnetic simulation results across the frequency range

────────────────────────────────────────────────────────────────────────────────

Notebook: TDK_Core_Data_to_CST.ipynb